1. Identification

| 1.1. Product identifier |  |  |
| --- | --- | --- |
| Product Identity | COM KOIL |  |
| Alternate Names | 90-152, Blended Formula, Com Koil Heavy Duty Acid Based External Condenser Coil Cleaner & Brightener- (Pink)- 55 gallon |  |
| 1.2. Relevant identified uses of the substance or mixture and uses advised against |  |  |
| Intended use | It is used as super concentrated acid based external condenser coil cleaner & brightener. |  |
| Application Method | Read all precautions and instructions carefully before and after use. |  |
| 1.3. Details of the supplier of the safety data sheet |  |  |
| Company Name | ComStar International Inc. |  |
|  | 20-47 128th Street, |  |
|  | College Point, NY 11356 |  |
| Telephone No. | 718-445-7900 800-328-0142 Fax: 718-353-5998 |  |

Emergency 24 HR response No:           1-800-424-9300 & 703-527-3887 CHEMTREC

Note: The CHEMTREC phone number is only for emergencies involving spills, leaks, fire, exposure or accident. Please direct all other inquiries to our customer service phone number.

2. Hazard(s) identification

Classification

This chemical is considered hazardous by the 2012 OSHA Hazard Communication Standard (29 CFR 1910.120

2.1. Classification of the substance or mixture

Corrosive to metals category 1; H290 Acute oral toxicity category 2; H300 Acute dermal toxicity category 1; H310 Skin Corrosion/Irritation category 1 A; H314 Serious Eye Damage/Eye Irritation Category 1; H318 Acute Inhalation Toxicity – Vapors category 2; H330 Specific target organ toxicity (single exposure) category 3; H335 Target Organs - Respiratory system

2.2. Label elements

Using the Toxicity Data listed in section 11 and 12 the product is labeled as follows.

SIGNAL WORD Warning

| Hazard Statements |  |
| --- | --- |
| H290 May be corrosive to metals |  |
| H300 Fatal if swallowed |  |
| H310 Fatal in contact with skin |  |
| H314 Causes severe skin burns and eye damage |  |
| H318 Causes serious eye damage |  |
| H330 Fatal if inhaled |  |
| H335 May cause respiratory irritation |  |
| Precautionary Statements |  |
| [Prevention]: |  |
| Wash face, hands, and any exposed skin thoroughly after handling |  |
| Do not eat, drink, or smoke when using this product |  |
| Do not get in eyes, on skin, or on clothing |  |
| Wear protective gloves/protective clothing/eye protection/face protection |  |
| Do not breathe dust/fume/gas/mist/vapors/spray |  |
| Use only outdoors or in a well-ventilated area |  |
| Wear respiratory protection |  |
| Keep only in original container |  |
| [Response]: |
| Immediately call a poison center or doctor/physician |
| Inhalation |
| If inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing |
| Skin |
| Wash contaminated clothing before reuse |
| If on skin (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower |
| Eyes |
| If in eyes: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. |
| Ingestion |
| Rinse mouth |
| Do not induce vomiting |
| Spills |
| Absorb spillage to prevent material damage |
| Storage |
| Store locked up |
| Store in a well-ventilated place. Keep container tightly closed |
| Store in corrosive resistant polypropylene container with a resistant in liner |
| Store in a dry place |
| Disposal |
| Dispose of contents/container to an approved waste disposal plant |
| Hazards not otherwise classified (HNOC) |
| None identified |

3. Composition/information on ingredients

| This product contains the following substances that present a hazard within the meaning of the relevant State and Federal Hazardous Substances regulations. |  |  |  |
| --- | --- | --- | --- |
| Ingredient/Chemical Designations | Weight % | GHS Classification | Notes |
| HYDROFLUORIC ACID CAS#: 7664-39-3 | <40 | Acute toxicity 3 Skin corrosion/irritation 1 Eye damage/ eye irritation 1 |  |
| PHOSPHORIC ACID CAS#: 7664-38-2 | >20 | Skin Corr. 1B, H314 Eye Dam. 1, H318 |  |
| SURFACTANT CAS#: 9016-45-9 | <10 | Skin Corrosion/Irritation: 3 Eye Damage/Irritation: 2B |  |
| WATER CAS#: N/A | Balance | Not Classified |  |

In accordance with paragraph (i) of §1910.1200, the specific chemical identity and/or exact percentage (concentration) of composition has been withheld as a trade secret. *The full texts of the phrases are shown in Section 16.

4. First aid measures

| 4.1. Description of first aid measures |  |
| --- | --- |
| General | Immediate and specialized first aid and medical treatment is required. Speed is of the essence. Flush with plenty of water immediately. Continue flushing during transport to hospital or medical center. |
| Inhalation | If not breathing, give artificial respiration. Do not use mouth-to-mouth method if victim ingested or inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper respiratory medical device. Remove to fresh air. Immediate medical attention is required. A nebulized solution of 2.5%Calciumgluconate may be administered with Oxygen by inhalation |
| Eyes | Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. In the case of contact with eyes, rinse immediately with plenty of water and seek medical advice. |
| Skin | Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required. Dermal burns may be treated with calcium gluconate gel or slurry in water or glycerin. This compound binds the active fluorides in an insoluble form and limits burn extension and pain. Soaking or immersion with iced 0.13% Benzalkonium chloride solution may be used for skin burns and should be continued until the pain is relieved. Do not use in eyes. |
| Ingestion | Do not induce vomiting. Call a physician or poison control center immediately. |

| 4.2. Most important symptoms and effects, both acute and delayed |  |
| --- | --- |
| Overview | Causes burns by all exposure routes. Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate tissue and danger of perforation |
| Notes to Physician | Treat symptomatically |

5. Fire-fighting measures

Suitable Extinguishing Media:   Water spray, carbon dioxide (CO2), dry chemical, alcohol-resistant foam.

Unsuitable Extinguishing Media: No information available

Flash Point:  No information available

Method:  No information available

Autoignition Temperature: No information available

Explosion Limits:

Upper: No data available

Lower: No data available

Sensitivity to Mechanical Impact:  No information available

Sensitivity to Static Discharge:  No information available

Specific Hazards Arising from the Chemical:

The product causes burns of eyes, skin, and mucous membranes. Thermal decomposition can lead to release of irritating gases and vapors.

Hazardous Combustion Products:

Gaseous hydrogen fluoride (HF).

Protective Equipment and Precautions for Firefighters:

As in any fire, wear self-contained breathing apparatus pressure-demand, MSHA/NIOSH (approved or equivalent) and full protective gear. Thermal decomposition can lead to release of irritating gases and vapors.

NFPA

Health                 Flammability          Instability        Physical hazards

4                                0                          1                         N/A

6. Accidental release measures

6.1. Personal precautions, protective equipment, and emergency procedures

Use personal protective equipment as required. Ensure adequate ventilation. Evacuate personnel to safe areas. Keep people away from and upwind of spill/leak.

6.2. Environmental precautions

Should not be released into the environment.

6.3. Methods and material for containment and cleaning up

Soak up with inert absorbent material and dispose of as hazardous waste. Keep in suitable, closed containers for disposal.

7. Handling and storage

7.1. Precautions for safe handling

Wear personal protective equipment/face protection. Do not get in eyes, on skin, or on clothing. Use only under a chemical fume hood. Do not breathe mist/vapors/spray. Do not ingest. If swallowed then seek immediate medical assistance.

7.2. Conditions for safe storage, including any incompatibilities

Keep containers tightly closed in a dry, cool, and well-ventilated place. Corrosives area. Do not store in metal containers. Incompatible Materials. Metals. Cyanides. Sulfides. Bases. Fluorine.

8. Exposure controls and personal protection

8.1. Control parameters

Exposure

| CAS No. | Ingredient | Source | Value |
| --- | --- | --- | --- |
| 7664-39-3 | HYDROFLUORIC ACID | OSHA PEL | (Vacated) TWA: 3 ppm (Vacated) TWA: 2.5 mg/m3 (Vacated) STEL: 6 ppm TWA: 3 ppm |
|  |  | ACGIH TLV | TWA: 0.5 ppm TWA: 2.5 mg/m3 Ceiling: 2 ppm Skin |
|  |  | NIOSH IDLH | IDLH: 30 ppm IDLH: 250 mg/m3 TWA: 3 ppm TWA: 2.5 mg/m3 Ceiling: 6 ppm Ceiling: 5 mg/m3 |
|  |  | Mexico OEL (TWA) | TWA: 0.5 ppm TWA: 2.5mg/m3 Ceiling: 2 ppm |
| 7664-38-2 | PHOSPHORIC ACID | OSHA | 1 mg/m3 |
|  |  | ACGIH | 1 mg/m3 |
|  |  | NIOSH | No Established Limit |
|  |  | Mexico OEL (TWA) | No Established Limit |
| 9016-45-9 | SURFACTANT | OSHA | 150 ppm |
|  |  | ACGIH | 150 ppm |
|  |  | NIOSH | No Established Limit |
|  |  | Mexico OEL (TWA) | No Established Limit |
| N/A | WATER | OSHA | No Established Limit |
|  |  | ACGIH | No Established Limit |
|  |  | NIOSH | No Established Limit |
|  |  | Mexico OEL (TWA) | No Established Limit |

| Carcinogen Data |  |  |  |
| --- | --- | --- | --- |
| CAS No. | Ingredient | Source | Value |
| 7664-39-3 | HYDROFLUORIC ACID | OSHA | Select Carcinogen: No |
|  |  | NTP | Known: No; Suspected: No |
|  |  | IARC | Group 1: No; Group 2a: No; Group 2b: No; Group 3: Yes; Group 4: No; |
| 7664-38-2 | PHOSPHORIC ACID | OSHA | Select Carcinogen: No |
|  |  | NTP | Known: No; Suspected: No |
|  |  | IARC | Group 1: No; Group 2a: No; Group 2b: No; Group 3: No; Group 4: No; |
| 9016-45-9 | SURFACTANT | OSHA | Select Carcinogen: No |
|  |  | NTP | Known: No; Suspected: No |
|  |  | IARC | Group 1: No; Group 2a: No; Group 2b: No; Group 3: No; Group 4: No; |
| N/A | WATER | OSHA | Select Carcinogen: No |
|  |  | NTP | Known: No; Suspected: No |
|  |  | IARC | Group 1: No; Group 2a: No; Group 2b: No; Group 3: No; Group 4: No; |

Legend

ACGIH - American Conference of Governmental Industrial Hygienists

OSHA - Occupational Safety and Health Administration

NIOSH IDLH: NIOSH - National Institute for Occupational Safety and Health

| 8.2. Exposure controls |  |
| --- | --- |
| Respiratory | Follow the OSHA respirator regulations found in 29 CFR 1910.134 or European Standard EN 149. Use a NIOSH/MSHA or European Standard EN 149 approved respirators if exposure limits are exceeded or if irritation or other symptoms are experienced. |
| Eyes/face protection | Tightly fitting safety goggles. Face protection shield |
| Skin and body protection | Wear appropriate protective gloves and clothing to prevent skin exposure. |
| Engineering Controls | Use only under a chemical fume hood. Ensure adequate ventilation, especially in confined areas. Ensure that eyewash stations and safety showers are close to the work station location. |
| Hygiene Measures | Handle in accordance with good industrial hygiene and safety practice. |

9. Physical and chemical properties

| Appearance | Pink Liquid |  |
| --- | --- | --- |
| Odor | Sharp |  |
| Odor threshold | No information available |  |
| pH | <1.0 |  |
| Melting point / freezing point | -35 °C / -31 °F |  |
| Initial boiling point and boiling range | 105 °C / 221 °F |  |
| Flash Point | No information available |  |
| Evaporation rate (Ether = 1) | No information available |  |
| Flammability (solid, gas) | Not Applicable |  |
| Upper/lower flammability or explosive limits | Lower Explosive Limit: No data available |  |
|  | Upper Explosive Limit: No data available |  |
| Vapor pressure (Pa) | No information available |  |
| Vapor Density | 2.21 |  |
| Specific Gravity | 1.15-1.20 |  |
| Solubility in Water | miscible |  |
| Partition coefficient n-octanol/water (Log Kow) | No data available |  |
| Auto-ignition temperature | No information available |  |
| Decomposition temperature | No information available |  |
| Viscosity (cSt) | No information available |  |
| Molecular Formula | H F |  |
| Molecular Weight | 20 |  |
| 9.2. Other information |  |  |
| No other relevant information. |  |  |

10. Stability and reactivity

10.1. Reactivity

None known, based on information available

10.2. Chemical stability

Stable under normal circumstances.

10.3. Possibility of hazardous reactions

Corrosive to metals. Contact with metals may evolve flammable hydrogen gas.

10.4. Conditions to avoid

Incompatible products. Excess heat.

10.5. Incompatible materials

Metals, Cyanides, Sulfides, Bases, Fluorine

10.6. Hazardous decomposition products

Gaseous hydrogen fluoride (HF)

10.7. Hazardous Polymerization

Hazardous polymerization does not occur.

11. Toxicological information

Acute Toxicity

Product Information

Oral LD50                           Category 2. ATE = 5 - 50 mg/kg.

Dermal LD50                      Category 1. ATE < 50 mg/kg.

Vapor LC50                        Category 2. ATE = 0.5 - 2 mg/l.

| Ingredient | LD50, ORAL | LD50, DERMAL | LC50, INHALATION |
| --- | --- | --- | --- |
| HYDROFLUORIC ACID (7664-39-3) | No data available | No data available | LC50 =0.79 mg/L(Rat)1 h |
| PHOSPHORIC ACID (7664-38-2) | 1530 mg/kg Rat | 2730 mg/kg Rabbit | No data available |
| SURFACTANT (9016-45-9) | 1310 mg/kg; | 2000 mg/kg | No data available |
| WATER (N/A) | No data available | No data available | No data available |

Toxicologically Synergistic Products   No information available

Delayed and immediate effects as well as chronic effects from short and long-term exposure

Irritation Causes severe burns by all exposure routes

Sensitization       No information available

Carcinogenicity

The table below indicates whether each agency has listed any ingredient as a carcinogen.

| Component | CAS No | IARC | NTP | ACGIH | OSHA | Mexico |
| --- | --- | --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 | Not listed | Not listed | Not listed | Not listed | Not listed |
| PHOSPHORIC ACID | 7664-38-2 | Not listed | Not listed | Not listed | Not listed | Not listed |
| SURFACTANT | 9016-45-9 | Not listed | Not listed | Not listed | Not listed | Not listed |
| Water | N/A | Not listed | Not listed | Not listed | Not listed | Not listed |

Mutagenic Effects               No information available

Reproductive Effects           No information available

Developmental Effects        No information available

Teratogenicity                     No information available

STOT - single exposure      Respiratory system

STOT - repeated exposure None known

Aspiration hazard                No information available

Symptoms / effects, both acute and delayed

Product is a corrosive material. Use of gastric lavage or emesis is contraindicated. Possible perforation of stomach or esophagus should be investigated: Ingestion causes severe swelling, severe damage to the delicate tissue and danger of perforation

Endocrine Disruptor Information

Other Adverse Effects

The toxicological properties have not been fully investigated.

12. Ecological information

12.1. Ecotoxicity

Do not empty into drains.

Aquatic Ecotoxicity

| Component | Freshwater Algae | Freshwater Fish | Microtox | Water Flea |
| --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID (7664-39-3) | Not Listed | LC50 = 660 mg/L, 48 | Not Listed | EC50 = 270 mg/L, 48h(Daphnia species) |
| PHOSPHORIC ACID (7664-38-2) | 138 mg/l | Not Available | Not Available | Not Available |
| SURFACTANT (9016-45-9) | Not Available | Not Available | Not Available | Not Available |
| WATER (N/A) | Not Available | Not Available | Not Available | Not Available |

12.2. Persistence and degradability

Soluble in water Persistence is unlikely based on information available. Miscible with water

12.3. Bioaccumulative potential

No information available.

12.4. Mobility in soil

Will likely be mobile in the environment due to its water solubility.

12.5. Results of PBT and vPvB assessment

This product contains no PBT/vPvB chemicals.

12.6. Other adverse effects

No data available.

| Component | log Pow |
| --- | --- |
| HYDROFLUORIC ACID (7664-39-3) | -1.4 |
| PHOSPHORIC ACID (7664-38-2) | -- |
| SURFACTANT (9016-45-9) | -- |
| WATER (N/A) | -- |

13. Disposal considerations

13.1. Waste treatment methods

Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Chemical waste generators must also consult local, regional, and national hazardous waste regulations to ensure complete and accurate classification

| Component | RCRA - U Series Wastes | RCRA - P Series Wastes |
| --- | --- | --- |
| Hydrogen fluoride - 7664-39-3 | U134 | -- |

14. Transport information

DOT

UN-No                                 UN1790

Proper Shipping Name      HYDROFLUORIC ACID

Hazard Class                      8

Subsidiary Hazard Class   6.1

Packing Group                   II

TDG

UN-No                               UN1790

Proper Shipping Name    HYDROFLUORIC ACID

Hazard Class                     8

Subsidiary Hazard Class  6.1

Packing Group                  II

IATA

UN-No                                 UN1790

Proper Shipping Name      HYDROFLUORIC ACID

Hazard Class                     8

Subsidiary Hazard Class  6.1

Packing Group                  II

IMDG/IMO

UN-No                               UN1790

Proper Shipping Name    HYDROFLUORIC ACID

Hazard Class                    8

Subsidiary Hazard Class 6.1

Packing Group                 II

15. Regulatory information

United States of America Inventory

| Component | CAS No | TSCA | TSCA Inventory notification - Active-Inactive | TSCA - EPA Regulatory Flags |
| --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 | X | ACTIVE | -- |
| PHOSPHORIC ACID | 7664-38-2 | X | X | -- |
| SURFACTANT | 9016-45-9 | X | X | -- |
| WATER | N/A | X | X | -- |

Legend:

TSCA US EPA (TSCA) - Toxic Substances Control Act, (40 CFR Part 710)

X - Listed

'-' - Not Listed

TSCA 12(b) - Notices of Export Not applicable

International Inventories

Canada (DSL/NDSL), Europe (EINECS/ELINCS/NLP), Philippines (PICCS), Japan (ENCS), Japan (ISHL), Australia (AICS), China (IECSC), Korea(KECL).

| Component | CAS No | DSL | NDSL | EINECS | PICCS | ENCS | ISHL | AICS | IECSC | KECL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 | X | -- | 231-634-8 | X | X | X | X | X | KE-20198 |
| PHOSPHORIC ACID | 7664-38-2 | X | -- | X | X | X | X | X | X | X |
| SURFACTANT | 9016-45-9 | X | -- | X | X | X | X | X | X | X |
| WATER | N/A | X | -- | X | X | X | X | X | X | X |

KECL - NIER number or KE number (http://ncis.nier.go.kr/en/main.do)

U.S. Federal Regulations

SARA 313

| Component | CAS No | Weight% | SARA 313 – Threshold Values % |
| --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 |  | 1.0 |
| PHOSPHORIC ACID | 7664-38-2 |  | X |
| SURFACTANT | 9016-45-9 |  | X |
| WATER | N/A |  | x |

SARA 311/312 Hazard Categories See section 2 for more information

CWA(Clean Water Act)

| Component | CWA - Hazardous Substances | CWA - Reportable Quantities | CWA - Toxic Pollutants | CWA - Priority Pollutants |
| --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | X | 100 lb | -- | -- |
| PHOSPHORIC ACID | X | -- | -- | -- |
| SURFACTANT | X | -- | -- | -- |
| WATER | X | -- | -- | -- |

Clean Air Act

| Component | HAPS Data | Class 1 Ozone Depletors | Class 2 Ozone Depletors |
| --- | --- | --- | --- |
| HYDROFLUORIC ACID | X | -- | -- |
| PHOSPHORIC ACID | X | -- | -- |
| SURFACTANT | X | -- | -- |
| WATER | X | -- | -- |

OSHA - Occupational Safety and Health Administration    Not applicable

| Component | Specifically Regulated Chemicals | Specifically Regulated Chemicals |
| --- | --- | --- |
| HYDROFLUORIC ACID | -- | TQ: 1000 lb |
| PHOSPHORIC ACID | -- | -- |
| SURFACTANT | -- | -- |
| WATER | -- | -- |

CERCLA      This material, as supplied, contains one or more substances regulated as a hazardous substance under the Comprehensive Environmental Response Compensation and Liability Act (CERCLA) (40 CFR 302)

| Component | Hazardous Substances RQs | CERCLA EHS RQs |
| --- | --- | --- |
| HYDROFLUORIC ACID | 100 lb | 100 lb |
| PHOSPHORIC ACID | -- | -- |
| SURFACTANT | -- | -- |
| WATER | -- | -- |

California Proposition 65               This product does not contain any Proposition 65 chemicals.

U.S. State Right-to-Know Regulations

| Component | Massachusetts | New Jersey | Pennsylvania | Illinois | Rhode Island |
| --- | --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | X | X | X | X | X |
| PHOSPHORIC ACID | X | X | X | X | X |
| SURFACTANT | X | X | X | X | X |
| WATER | X | X | X | X | X |

U.S. Department of Transportation

Reportable Quantity (RQ):       Y

DOT Marine Pollutant              N

DOT Severe Marine Pollutant  N

U.S. Department of Homeland Security

This product contains the following DHS chemicals: Legend - STQs = Screening Threshold Quantities, APA = A placarded amount

| Component | DHS Chemical Facility Anti-Terrorism Standard |
| --- | --- |
| HYDROFLUORIC ACID | Release STQs - 1000lb (concentration >=50%) Release STQs - 1000lb (anhydrous) Theft STQs - 45lb (anhydrous) |
| PHOSPHORIC ACID | -- |
| SURFACTANT | -- |
| WATER | -- |

Other International Regulations

Mexico – Grade            No information available

Authorisation/Restrictions according to EU REACH

| Component | REACH (1907/2006) - Annex XIV - Substances Subject to Authorization | REACH (1907/2006) - Annex XVII - Restrictions on Certain Dangerous Substances | REACH Regulation (EC1907/2006) article 59 – Candidate List of Substances of Very High Concern (SVHC) |
| --- | --- | --- | --- |
| HYDROFLUORIC ACID | -- | Use restricted. See item 75. (see link for restriction details) | -- |
| PHOSPHORIC ACID | -- | -- | -- |
| SURFACTANT | -- | -- | -- |
| WATER | -- | -- | -- |

Safety, health, and environmental regulations/legislation specific for the substance or mixture

| Component | CAS No | OECD HPV | Persistent Organic Pollutant | Ozone Depletion Potential | Restriction of Hazardous Substances (RoHS) |
| --- | --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 | Listed | Not applicable | Not applicable | Not applicable |
| PHOSPHORIC ACID | 7664-38-2 | Listed | Not applicable | Not applicable | Not applicable |
| SURFACTANT | 9016-45-9 | Listed | Not applicable | Not applicable | Not applicable |
| WATER | N/A | Listed | Not applicable | Not applicable | Not applicable |

| Component | CAS No | Seveso III Directive (2012/18/EC) - Qualifying Quantities for Major Accident Notification | Seveso III Directive (2012/18/EC) - Qualifying Quantities for Safety Report Requirements | Rotterdam Convention (PIC) | Basel Convention (Hazardous Waste) |
| --- | --- | --- | --- | --- | --- |
| HYDROFLUORIC ACID | 7664-39-3 | Not applicable | Not applicable | Not applicable | Annex I - Y34 |
| PHOSPHORIC ACID | 7664-38-2 | Not applicable | Not applicable | Not applicable | Not applicable |
| SURFACTANT | 9016-45-9 | Not applicable | Not applicable | Not applicable | Not applicable |
| WATER | N/A | Not applicable | Not applicable | Not applicable | Not applicable |

16. Other information

The information and recommendations contained herein are based upon data believed to be correct. However, no guarantee or warranty of any kind, expressed or implied, is made with respect to the information contained herein. We accept no responsibility and disclaim all liability for any harmful effects which may be caused by exposure to our products. Customers/users of this product must comply with all applicable health and safety laws, regulations, and orders.

The opinions expressed are those of qualified experts within ComStar International Inc. We believe that the information contained is current as of the date of the Safety Data Sheet. Since the use of this information and of these opinions and the conditions of the use of the product are not within the control of ComStar International Inc., it is the user’s obligation to determine the conditions of safe use of the product.

End of Document